The Navigator System Quick Start Guide

This document describes the basic set-up required to get The Navigator up and running … or sailing.

# UNZIP The Files

Three code files exist, GPS.asm, PWM.asm, and Nav.c  .  They were packed using WINZIP.  UnZIP these files into appropriate folders.

# Assemble and Compile the Files

All the code was developed under Microchip MPLAP, V8.20.  You can either create MPLAB projects for the code, or quick build them under MPLAB.  GPS and PWM are straight assembly (.asm) files for the PIC16F876A, and NAV is a “C” (.c) file, compiled with the Microchip C18 compiler.  It is a single module containing all functions as well as the “main” function.  The .hex files produced may then be loaded into your project boards by whatever method you desire.  The NAV.C module is targeted to a PIC18F2525.

# Initializing The System

# GPS Board and Sensor Unit

The GPS Board requires raw navigation information from a sensor unit.  I use a Garmin GPS-15 h/l sensor, however ANY sensor will work that outputs the $GPGGA sentence at 4800, 8, N,1.  Using Garmin’s Sensorconfig utility ( a free download), configure the sensor to output $GPGGA.

The RS-232 output of the sensor board drives the GPS Board.  Only the output of the sensor is needed, considering a 9-pin RS-232 connector, the output of the sensor is Pin 2 which is connected to the RS-232 input of the GPS Board, Pin 3.  The ground in each case is Pin 5.

Once the sensor unit and GPS board are powered-up, the LCD display will indicate an initial massage GPS Start.  A second later, the message NF is displayed, indicating “No Fix”.  Following successful GPS lock-up, the LCD will display the partial latitude coordinates in Line 1, and the partial longitude coordinates in Line 2.  The yellow LED should be full ON, and the upper and middle red LEDs will blink at a one-second rate.

The coordinate displays indicate the following:  For both lat and lon, the full coordinate minutes are displayed in the form mm.mmmm followed by the processed coordinates in the form m.mmmm.  The first data on each line is the actual GPS fix info, minus the whole degree portion of your position.  The second field on each line is the processed fix, showing only the least significant whole minute and the fraction.  Processing consists of an eight-slot “moving average” filter, originally intended to smooth any radical glitches in the received GPS data.  This proved unnecessary; consequently the second field on each line reflects the actual, real-time GPS fix.  If the moving average filter IS desired, it can be enabled by conditional reassembly of the code, following the commented instructions in GPS.asm.  When the MA filter is ON, depressing the S1 button on the LCD board resets the moving average – you can observe the average forming over eight seconds on the LCD.  Finally, the number 1 or 2 will appear at the end of the longitude Line 2 display.  This reflects the GPS “quality” of fix.  A “1” indicates a usable fix without WAAS refinement.  A “2” indicates that WAAS is enabled.  (The GPS sensor needs to “see” the WAAS satellite, which is positioned roughly over the Gulf of Mexico in the eastern USA – foliage or terrain at your specific location may prevent a good WAAS fix.)

In any case, the lat/lon display on the LCD indicates that the sensor is providing valid fix information to the GPS Board, and to the NAV Board.

Finally, when the entire system is ready, depressing the S2 button on the GPS LCD board will cause the GPS Board to attempt to “sync-up” to the NAV Board, allowing controlled position information to pass to the NAV Board.  If the NAV Board is not ready to be synchronized, depressing the S2 button will result in the center red LED to stop blinking and lock hard ON, indicating a sync failure.  When the NAV Board IS ready, depressing S2 results in the top red LED to lock ON, indicating a successful sync handshake.  Go ahead and depress S2 now – but restart the GPS Board later!

A functioning GPS Board display will look like this:

44.9000  4.9000			(with YOUR position of course!)

  - 7.0000  2

# Navigation Board Initialization

Now let’s set-up the Navigation Board.  It is assumed that you have already planned a trip consisting of a start waypoint and multiple legs defined by a north-based heading and distance in nautical miles … refer to the Users Guide for details.

OK – now we need a laptop or desktop system running a terminal program like Hyperterminal or TeraTermPro.  Parameters are 19200, 8, N, 1  .  A three-wire RS-232 serial connection from the terminal to the NAV Board is required.  ONLY (important) three wires are necessary, TX data, RX data, and ground.  Using a complete serial cable will cause errors, as the other pins on the board are used for serial programming the PIC.

Now you need to know the exact latitude/longitude of the position where your trip starts – a dock, corner of a field, etc.  Set your handheld GPS unit to display position is the ddmm.mmmm format – or if you are really hardcore, determine the position from a good chart or topo map!  Note both the FULL lat and lon coordinates defining your start point.  For example, here is a sample reading from my GPS unit:

  - 07316.4174

This base data needs to be entered into The Navigator – the next software revision will eliminate this step.  Right now, the GPS Board only provides the least significant “m.mmmm” data to the NAV Board.  The balance of the coordinates are preset in the NAV Board.  You only need to do this once as long as the start point of a trip remains the same … once entered, the data is instantly recalled from EEPROM for the same start point.

So, knowing your start point, power on the NAV Board with the serial connection ready.

You will see the following on the LCD:

NAVIGATION

S1 : PC  BASE      S2 :  EE

Depress the S1 button, and observe the prompt on the PC:

Flat  :  mm		This is requesting the BASE mm of your trip start latitude.

The BASE is whole decimal mm number (disregard the frational part )  MINUS the one’s position.  Using our example of 4344.9830, the BASE is mm (44) minus one’s position (4) or 40 .

So, enter 40    - when running, the system reconstructs the entire real-time coordinate … recall that only the m.mmmm data is passed from the GPS!  Now enter a return (cr), and observe:

Lat as dd	This is easier – simply enter the dd (whole degrees) of the position, for example, 43 (cr).  Observe:

Flon  :  mm	Enter the BASE longitude, using the method employed for latitude.

Thus, using our example, enter mm (16) minus one’s position (6), thus 16-6, or   10 (cr).  Finally, observe :

Lon as dd	Enter the whole dd longitude, in our example 073 (or just 73)  (cr)

NOTE:  As I indicated, this business will disappear in Version 2.0 of the software.

The GPS Board was part of another project, and pressed into service “as-was” for this Navigation project – I just didn’t want to rewrite all the synchronization stuff in the GPS module while concentrating on the Navigation code.  HOWEVER, a hidden advantage in the above as you, the user, can “trick” the entire system while refining and debugging a round trip.  By modifying the actual base lat/lon values entered above, one can make the system think it is starting at a different point, like in you shop on the bench!  That way, you can observe how subtle differences in position affect the off-course corrections.

Finally, for future runs from the same start point, the data just entered is stored in EEPROM for instant recall.  Just depress S2 to recall previously entered home-base coordinates!

SO, following that final (cr), (or S2 button depression) observe that the LCD on the Navigation Board now indicates:

NAVIGATION

S1 :  PC  TRIP          S2 : EE

The following message will appear on the PC terminal:

-----

Enter Trip Parameters - ? to reenter parameter

Seg Cnt?

Now we enter the details of a trip.  If a typing mistake is made, enter a “?” to reenter the given data.  “Seg Cnt?” requests the total number of legs in the trip.  Right now, the maximum number of legs is 4 that can be increased by recompiling Nav.c, and following the commented instructions.  So let’s enter 4 (cr)

Observe:

Dock Lat?	Enter the start position latitude as decimal degrees.  From our example of 4344.9830, take the mm.mmmm component of the latitude and divide by 60.0 to produce 0.74971, and then add the whole degree value of 43.0, resulting in 43.74971.  Enter that value to five fractional digits.  (cr)  Then observe:

Dock Lon?	Do the same as above for longitude.  Then,

Dock HDG?	Now we are at the point of entering our trip, leg by leg.  For this example, we will run a course consisting of 4 legs forming a square, ending where we started!  For REAL simplicity, we will leave the dock on a heading (True North based) of north, or zero degrees, travel 1.0 nautical miles, then turn right to a heading of east (90.0 degrees) for 1.0 nm, then south (180.0 degrees) for 1.0 nm, finally turning to 270.0 degrees for a nm to return home.  So, enter the first leg heading of 0.0 (cr).  Then, see :

Dock NM?	This is the distance run on a heading of north, enter 1.0 (cr).

Then see :

1 Leg HDG?	This is the heading of the eastbound leg – enter 90.0  (cr)

1 Leg NM?	The distance of the eastbound leg … 1.0 nm.  Enter 1.0 (cr)

You will be prompted for the heading and distance of EACH subsequent leg.  Notice

while entering information, LCD Line 2 will update with a leg number as each element is entered.  Finally, after all defined leg info has been entered, observe:

Rev?  Y=1	Most hobby R/C systems allow a given servo to be “reversed” in motion, depending on the specific gear and application.  If YOUR steering control servo requires reversal, enter a “1” in response, else; enter “0”.

Now, observe the NAV LCD – it should indicate the following:

S1 To Calc Trip

Originally, when developing the navigation algorithms, I assumed a round trip would

terminate at the start point, and had code that “ran” the trip internally to see if this was the case – well, now you can terminate wherever, and that code is disabled.  So depressing the S1 button now causes the system to calculate internal turn waypoints, etc. and build the run-time structures.  So go ahead and press S1, and see the following:

S1 To Calc Trip

S2 For GPS Sync

Assuming a functional GPS Board and sensor, FIRST depress the S2 button on the Navigation Board.  The LCD will blank, and the bottom, yellow LED will begin to flash, indicating the main loop is waiting for sync commands from the GPS Board.  Second, depress the S2 button on the GPS Board.  The TOP, red LEDs on both NAV and GPS Boards will come hard ON, indicating system synchronism.  The following will appear on the NAV LCD:

MA m.mmmm  m.mmmm

S3  TO  NAVIGATE

Line 1 is the real-time position showing only the one’s position and fraction … recall that the system synthesizes the full GPS position based on the set-up conditions.  If the entire platform is now moved, the Line 1 display will update once every four seconds to the real-time position.  Assuming that you are now ready to go roving (or sailing), depress the S3 button on the NAV board and observe the following:

MA m.mmmm  m.mmmm

X  0NM  -1M  L10

Line 2 constantly indicates the status of the run in real-time.  Probably for your first run or so, you will want to carry the robot or platform around the course to see how the trip develops.  I carried Mr. Darby, my faithful large-scale tugboat, around my lake in a 16’ fishing boat with small outboard to see how things went!  Anyway, Line 2 means the following:

X nNM – distance OFF COURSE (X) in nautical miles / 1000.0.  A nm is 1852 meters along a great circle arc, consider that a straight line for our purposes.  Consequently if our indication was X 1NM we are 1852/1000 m off leg center line, or 1.85 meters.  Right now, the system will cause a course correction when the X factor is 0.002 nm, or 3.7m away from leg center.  Again, a recompilation of nav.c can alter the trigger point for a correction.  If the nNM value is preceded by a minus (-) sign, we are LEFT of center and a correction to the right is required, no sign indicates that we are RIGHT of center, and a correction to the left is needed.

The next field on Line 2 indicates the off-course error in actual meters.  In the example above, we show ourselves being one meter to the left of desired center line.  Finally, L1 indicates we are running Leg 1.  That will update when executing a turn to a new leg at the end-of-leg waypoint.  Finally, the “0” indicates no steering algorithm activity is taking place.  The final digit in the display is a reflection of the state machine value in MY rudder displacement function.

SO!!!  Now we are Navigating.  As you walk (or putter around the lake) you can observe where you are in real-time in relation to the leg centerline.  When the “X Factor” bumps up to .002 nm or greater, the steering algorithm will begin to correct appropriately.  Check it out and have fun!  OH, at the END of the final leg, the center red LED on the NAV Board will lock ON hard, and in my case I put the rudder hard-over to cause the boat to circle in a tight radius turn.  Trip Over!

# PWM Board Initialization

After all that, the actual servo control PWM board is easy to set-up.  At any time following system start-up, observe the Heartbeat LED on the PWM Board blinking, and the yellow LED hard OFF.  The PWM Board supports a single steering servo with Radio Control (R/C) override.  Here’s how it works:  The steering servo and its power supply, usually an independent battery pack are connected to the Navigator’s PWM generator thru a normally closed relay.  Steering information from the NAV Board creates the appropriate PWM for steering the vehicle.  IF something fails, or you just panic, the PWM Board relay can be energized by grounding a lead, the source of the ground signal is left up to the user – I employ a contact closure module on a R/C setup, so when there is a need depressing a button on the TX closes a relay on the RX system, grounding the fail-safe lead.  When the relay is activated, the driver can control the steering with a standard R/C setup, as the RX steering PWM is switched to the steering servo, taking The Navigator out of the loop!  Has not happened yet, I’m glad to say!

Anyway, the PWM signals produced by the PWM Board are derived from a table in the software, based upon the center position of the servo in question.  Thus the only set-up required on the PWM board is locating servo center.  The potentiometer on the PWM Board drives an a/d channel on the PIC – before tripping, rotate the pot until your steering servo is exactly centered … then momentarily GROUND PORTB.3 on the PIC – I use a small PCB pushbutton.  The YELLOW LED on the PWM Board will instantly lock hard ON, indicating that the a/d center position value has been captured and the appropriate offset table has been generated.  That’s it!

FUTURE Navigator Revisions

As all these projects go, this one is never done!  Here’s what is in the works for the next Navigator revision.

- Consolidation of all functions on a single board.  I’m building the next system based on a PIC 18F TQFP part, all surface mount, coded entirely in CCS “C”.  It’s getting there as we type!

- Consolidate all GPS functions into the main code; eliminating the need to synchronize processes, and eliminating all the initial coordinate partial entry stuff.

- Allow the user to produce waypoints by “walking” a course and depressing an S button at each turn point … the system will calculate the leg distance stuff.

- Also working on a parallel system using the microMega FPU V3.1 math coprocessor that can parse GPS sentences as well as do the heavy floating-point lifting much faster than “C” on hardware without real floating-point hardware.

- Whatever else sounds cool.

## Contact Me

I hope someone out there actually builds this system – it works way better than I ever anticipated last winter when this thing started to gel.  Please feel free to e-mail me with any suggestions and comments.  I understand that the initial version of the UI is a bit clunky – but in my defense I created this thing using software and stuff I had around already and just integrated things that really didn’t fit is some cases…I tried to eliminate your need to understand some of the obscure navigation stuff and spherical trig algorithms…and just use the system.  My guess is that most of you that have read to this point are “up” on PICS and mobile platforms and don’t need or want a whole bunch of minutia regarding the innards…..see you!

John Molnar

crank2max@hotmail.com