void DoEscape(void)

// Define a bumper response escape behavior

{

static uint32_t ticker = 0;

static uint8_t state = 0;

static uint8_t dir = 0;

if ((mBumpers == 0) && (state ==0))

// Don't do anything if nothing to do

{

return;

}

if (priority <= escape)

{

priority = escape;

switch(state)

{

case 0: // record bumper, backup

dir = mBumpers;

ticker = millis() + 1000;  // 1 second

GoBackward();

GoSong();

state = 1;

Serial.print("Bumper dir: ");

Serial.println(dir,HEX);

break;

case 1:           // wait for backup done

if (ticker < millis())

{

state = 2;

Serial.println("Do turn");

}

break;

case 2:           // choose turn direction

if (dir == RIGHT)

{

Serial.println("spin left");

SpinLeft();

}

else if (dir == LEFT)

{

Serial.println("spin right");

SpinRight();

}

else

{

Serial.println("ahead");

SpinRight();

}

ticker = millis() + 1000;

state = 3;

break;

case 3:           // wait for turn done

if (ticker < millis())

{

state = 0;

dir = 0;

priority = idle;

Serial.println("All done");

}

break;

}

}

}

Listing 1

void InitTimer0(void)

{

/*

* Initialize timer0 to generate an output compare interrupt, and

* set the output compare register so that we get that interrupt

* every 10us.

*/

TIFR  |= _BV(OCIE0);

TCCR0  = _BV(WGM01)|_BV(CS01); /* CTC, prescale = 8 */

TCNT0  = 0;

TIMSK |= _BV(OCIE0);    	/* enable output compare interrupt */

OCR0   = 18;          	/* match in 10us */

}

SIGNAL(SIG_OUTPUT_COMPARE0)

/*

* 10 microsecond ISR

*/

{

t_10us++;

if (t_10us > 100)

{

t_1ms++;

t_10us = 0;

}

}

Listing 2

void BackSee(void)

{

static uint32_t 	dur=0;

static uint8_t 	bState=0;

uint8_t		whatSide = 0;

if (priority > BACKSEE)

return;// something more important happening elsewhere

if (priority != BACKSEE)

{

// This is new and more important

bState = 0;

if (irpd != 7)			// Someone near us!

{

priority = BACKSEE;	// Take over priority

bState = 1;

if ((irpd & BACKSIDE) == 0)

{

whatSide = BACKSIDE;

}

else if ((irpd & RIGHTSIDE) == 0)

{

whatSide = RIGHTSIDE;

}

else

{

whatSide = LEFTSIDE;

}

}

else

{

return;

}

}

// We are currently running

switch (bState)

{

case 1:	// pivot

if ((whatSide == BACKSIDE)  || (whatSide == LEFTSIDE))

{

rS = MSTFWD;

lS = MSTREV;

}

else

{

rS = MSTREV;

lS = MSTFWD;

}

dur = t_1ms + 2000;	// turn around

bState = 2;

break;

case 2:	// wait to complete, and drop priority

if (t_1ms > dur)

{

bState = 0;

priority = 0;	// done, clear out

}

break;

default:

break;

}

}

Listing 3

void Init(void)

/*

*	Set up all the things we want to play with.

*/

{

/*

* Our background timer, 1ms per count

* Turn T1 on

* Use internal 10MHz peripheral clock

* No prescale (1:1)

* period of 10,000 tics (10000 * 1/10MHz = 1ms)

*/

OpenTimer1(T1_ON | T1_SOURCE_INT | T1_PS_1_1, 10000);

ConfigIntTimer1(T1_INT_ON | T1_INT_PRIOR_2);

INTEnableSystemMultiVectoredInt();

}

void __ISR(_TIMER_1_VECTOR, ipl2) Timer1Handler(void)

{

// clear the interrupt flag

mT1ClearIntFlag();

t_1ms++;

}

Listing 4

while(1)

{

xStick = ((ADCRead(0)+0)/15)-7;		//Get x axis value

yStick = (15-((ADCRead(2)+0)/15))-7;	//Get y axis value

//Now figure motor values

leftMotor = yStick + xStick;

if (leftMotor>8)

leftMotor = 8;

if (leftMotor < -8)

leftMotor = -8;

//create a deadband

if ((leftMotor < 0) && (leftMotor > -2))

leftMotor = 0;

if ((leftMotor > 0) && (leftMotor < 2))

leftMotor = 0;

rightMotor= yStick - xStick;

if (rightMotor > 8)

rightMotor = 8;

if (rightMotor < -8)

rightMotor = -8;

//create a deadband

if ((rightMotor < 0) && (rightMotor > -2))

rightMotor = 0;

if ((rightMotor > 0) && (rightMotor < 2))

rightMotor = 0;

}

Listing 5