' {$STAMP BS2}

' {$PBASIC 2.5}

'~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

'========== assign pins

ReceivePin  PIN 0

SendPin     PIN 1

LeftMotor   PIN 12

RightMotor  PIN 13

RearBumper  PIN 7

RightBumper PIN 8

FrontBumper PIN 9

LeftBumper  PIN 10

'==================Declare necessary variable

SENSORS     VAR Nib

Direction   VAR Byte

T           VAR Byte

bData VAR Byte(2)    'Received data buffer from RobotBASIC

'byte 0 is the command code and byte 1

'is a parameter for the command.

bBuffer VAR Byte(5)  'Buffer of data to be sent to RobotBASIC

'byte 0 is the bumper sensor. Byte 1 is the

'infrared sensors. Byte 2 is the line sensors.

'Byte 3 and 4 is a 16 bit integer for the data

'to be returnd as the result of some commands.

'Byte 3 is the MSB, byte 4 is the LSB.

'See RobotBASIC help for more details.

'==============================================================

Main:

PAUSE 1000   'WAIT FOR the eb500 radio TO be ready

DO

SERIN ReceivePin,84,[STR bData\2] 'receive command and its

'parameter from RobotBASIC

IF bData(0) = 3  THEN

GOSUB rLocate

ELSEIF bData(0) = 6 THEN

Direction = 1

GOSUB rForward

ELSEIF bData(0) = 7 THEN

Direction = 0

GOSUB rForward

ELSEIF bData(0) = 12 THEN

Direction = 1

GOSUB rTurn

ELSEIF bData(0) = 13 THEN

Direction = 0

GOSUB rTurn

ELSEIF bData(0) = 24 THEN

GOSUB rCompass

ELSEIF bData(0) = 48 THEN

Direction = 1

GOSUB rLook

ELSEIF bData(0) = 49 THEN

Direction = 0

GOSUB rLook

ELSEIF bData(0) = 96 THEN

GOSUB rBeacon

ELSEIF bData(0) = 192 THEN

Direction = 1

GOSUB rRange

ELSEIF bData(0) = 193 THEN

Direction = 0

GOSUB rRange

ELSEIF bData(0) = 129 THEN

GOSUB rPen

ELSEIF bData(0) = 36 THEN

GOSUB rSpeed

ELSEIF bData(0) = 66 THEN

GOSUB rGPS

ELSE

GOSUB NotRecognized

ENDIF

SEROUT SendPin,84,[STR bBuffer\5]  'Send back 5 bytes to RobotBASIC

LOOP

END

'=============================================================

rLocate:

DEBUG "rLocate ",DEC bData(1),CR

GOSUB ReadSensors

RETURN

'=============================================================

rForward:

' DEBUG "rForward ",DEC bData(1),CR

' move the number of units specified

' by the variable bData(1)

FOR T = 1 TO bData(1)

' This code moves our robot about

' 1/4 inch

IF Direction = 1 THEN

PULSOUT LeftMotor,775

PULSOUT RightMotor,725

ELSE

PULSOUT LeftMotor,725

PULSOUT RightMotor,775

ENDIF

PAUSE 10

NEXT

GOSUB ReadSensors

RETURN

'=============================================================

rTurn:

' DEBUG "rTurn ",DEC bData(1),CR

' This subroutine will turn

' the number of units specified

' by the variable bData(1)

IF Direction = 1 THEN Direction = 30

FOR T = 1 TO  bData(1)

' The following code turns our

' robot about 1 degrees

PULSOUT LeftMotor,735+Direction

PULSOUT RightMotor,735+Direction

PAUSE 10

NEXT

GOSUB ReadSensors

RETURN

'==============================================================

rCompass:

DEBUG "rCompass ",DEC bData(1),CR

GOSUB ReadSensors

bBuffer(3) = 0

bBuffer(4) = 220    'value for testing

RANDOM bBuffer(4)

RETURN

'=============================================================

rLook:

DEBUG "rLook ",DEC bData(1),CR

GOSUB ReadSensors

bBuffer(3) = 0

bBuffer(4) = 210  'value for testing

RETURN

'=============================================================

rBeacon:

DEBUG "rBeacon ",DEC bData(1),CR

GOSUB ReadSensors

bBuffer(3) = 0

bBuffer(4) = 50   'value for testing

RETURN

'=============================================================

rRange:

DEBUG "rRange ",DEC bData(1),CR

GOSUB ReadSensors

bBuffer(3) = 0

bBuffer(4) = 100     'value for testing

RETURN

'=============================================================

rPen:

DEBUG "rPen ",DEC bData(1),CR

GOSUB ReadSensors

RETURN

'=============================================================

rSpeed:

DEBUG "rSpeed ",DEC bData(1),CR

GOSUB ReadSensors

RETURN

'=============================================================

rGPS:

DEBUG "rGPS ",DEC bData(1),CR

bBuffer(0) = 1

bBuffer(1) = 210     'values for testing

bBuffer(2) = 2

bBuffer(3) = 220     'values for testing

bBuffer(4) = 0

RETURN

'=============================================================

NotRecognized:

DEBUG "Not A Command ",DEC bData(0),"  ",DEC bData(1),CR

bBuffer(0) = 255   'set all returned values to -1

bBuffer(1) = 255

bBuffer(2) = 255

bBuffer(3) = 255

bBuffer(4) = 255

RETURN

'=============================================================

ReadSensors:

'read bumper sensors

SENSORS.BIT0 = IN7

SENSORS.BIT1 = IN8

SENSORS.BIT2 = IN9

SENSORS.BIT3 = IN10

bBuffer(0) = SENSORS

'read infrared sensors

'none for now

bBuffer(1) = 0

'read Line sensors

'none of now

bBuffer(2) = 0

'set the last two bytes to zero

bBuffer(3) = 0

bBuffer(4) = 0

RETURN

'=============================================================

Figure 4: The BS2 shell program for carrying out the communications protocol with RobotBASIC.