/////////////////////////////////////////////////////////////////////////////////////////////

//	 -Program to implement the I2C Communications to the Acroname (Devantech) Electronic

//	  R117 Compass. Written by Dennis Bogden using RobotC provided example code. January 11, 2008

//	 -Compass is connected as I2C address 0xC0 to initiate a read and to 0xC1 to read back value.

//	 -Compass is connected to sensor port 4 of the NXT brick.

/////////////////////////////////////////////////////////////////////////////////////////////

// Constants in the program

const tSensors			port = S4;			// NXT port compass is connected

const byte			ICADDRESS = 0xC0;	// Address for R117 to initiate read of the compass

// Variables in the program

byte	raw_compass_value=0;				//output goes from 0-126 and then -127 to -1

word	normalized_compass_value=0; 			//convert output to go 0-255

word	compass_degrees=0;					//convert output to read 0-359 degrees

short	ControlIn;						//Compass address

/////////////////////////////////////////////////////////////////////////////////////////////

//

//				 Wait for I2C Bus to Be Ready

//

/////////////////////////////////////////////////////////////////////////////////////////////

void waitForI2CBus()

{

TI2CStatus nStatus;

nStatus = nI2CStatus[port];

while (true)

{

nStatus = nI2CStatus[port];

switch (nStatus)

{

case NO_ERR:

return;

case STAT_COMM_PENDING:

nxtDisplayTextLine(2, "I2C Pending");

break;

}

}

}

//																					 Main Task

task main()

{

// I2C message configuration

byte i2cconfg[3];

const short Size		= 0;

const short Address		= 1;

const short Controlreg		= 2;

// In RobotC, under Motors and Sensors, the port S4 is defined as I2C

SensorType[port] = sensorI2CCustom;

Read_Compass_loop:							// Loop or repeat the program

ControlIn=0xC1;						//Address to read back data from compass

{

waitForI2CBus();

// Build outgoing message and send to activate the IC

i2cconfg[Size]			 = 2;			// Two bytes in the message

i2cconfg[Address]		 = ICADDRESS; 	// The compass initiate address (0xC0)

i2cconfg[Controlreg] = ControlIn;			// The user defined I/O ports

// INITIALIZE Compass

sendI2CMsg(port, i2cconfg[0] ,0);

waitForI2CBus();

//THEN-->

// WANT ONE BYTE REPLY FROM Compass

//

byte replyMessage[1];

// Build outgoing message and send

// One bytes in the message

i2cconfg[Size]			 = 1;

i2cconfg[Address]		 = ICADDRESS;

sendI2CMsg(port, i2cconfg[0], 1);

waitForI2CBus();

//READ THE RETURN VALUE FROM THE IC

readI2CReply(port, replyMessage[0], 1);

///////////////////////////////////////////////////////////////////////////////

//DISPLAY VALUE ON NXT DISPLAY

///////////////////////////////////////////////////////////////////////////////

//The raw_compass_value-->output goes from 0-126 and then -127 to -1

raw_compass_value=replyMessage[0];

if (raw_compass_value<0){

normalized_compass_value=raw_compass_value+256;	 //normalized_compass_value  0-255

}

else

normalized_compass_value=raw_compass_value;

compass_degrees=normalized_compass_value*1.418;	 //convert the normailed value to 0-

// 360 degrees

nxtDisplayTextLine(4, "%02d", (short) compass_degrees);   //Show compass degrees on NXT

// display

// A little delay between each I/O refresh

wait1Msec(10);

goto Read_Compass_loop;																				//REPEAT LOOP

}

}